Riya Vartha -24102B0032

Ayushi Jain – 24102B0002

TE CMPN B

A Scorecard-Based Assessment of E-Waste Management Performance in Himachal Pradesh

Compiled from the Himachal Pradesh State E-Waste Management Tracker (FY 2025-26 reporting cycle)

Data compiled in line with the E-Waste (Management) Rules, 2022 (CPCB, Government of India)

Abstract—This report presents a quantitative assessment of e-waste management performance in the state of Himachal Pradesh (HP), India, based on a district-level tracking instrument spanning generation, collection, formal recycling, regulatory compliance, informal-sector integration, environmental safeguards, and public awareness. Applying a 100-point weighted scorecard, the state records an aggregate score of 63.6, placing it in the “Needs Improvement” band. The dataset covers all 12 districts and reports a total annual e-waste generation of 530.3 tonnes, a collection rate of 36.4% of generated volume, extended producer responsibility (EPR) collection-target achievement of 94.0%, and four CPCB/SPCB-authorized recyclers concentrated entirely in Solan district. Regulatory compliance stands at 60%, informal-sector formalization remains incomplete across four identified hotspots, and awareness outreach has so far reached only four of twelve districts. The analysis identifies collection infrastructure in hill districts, recycler geographic concentration, EPR-penalty resolution, and informal-sector formalization as the principal levers for improving the state's standing.

Index Terms—Electronic waste, extended producer responsibility, e-waste collection, informal sector formalization, environmental monitoring, Himachal Pradesh, circular economy, CPCB E-Waste Rules 2022.

I. Introduction

The E-Waste (Management) Rules, 2022, issued by the Ministry of Environment, Forest and Climate Change and administered by the Central Pollution Control Board (CPCB), establish extended producer responsibility (EPR), authorization requirements for recyclers and dismantlers, and take-back obligations for electronic goods placed on the Indian market. Effective implementation, however, depends on continuous state-level monitoring of generation, collection, treatment, and compliance indicators.

This report analyzes a structured tracking workbook maintained for Himachal Pradesh, a hill state comprising twelve administrative districts. The workbook records district-wise generation inventories, collection logs, authorized-recycler infrastructure, EPR compliance status, informal-sector hotspots, environmental and health monitoring visits, and awareness programs, and aggregates these into a single weighted scorecard. The remainder of this report describes the scoring methodology (Section II), presents category-wise results (Section III), discusses cross-cutting findings (Section IV), proposes recommendations (Section V), and concludes (Section VI).

II. Methodology

The scorecard allocates 100 points across seven weighted categories: Generation & Inventory (15), Collection Performance (20), Formal Recycling & Treatment (20), Regulatory Compliance (15), Informal Sector Integration (10), Environmental & Health Safeguards (10), and Awareness & Outreach (10). For each category, an achieved value is compared against a user-defined benchmark/target to compute an achievement percentage, which is capped at 100% and multiplied by the category weight to yield the score earned. The composite score of 63.6/100 is classified using four bands: Excellent (85-100), Good (65-84), Needs Improvement (45-64), and Critical (0-44).

Underlying data are recorded at the district level across seven source sheets, with district names held consistent to permit cross-sheet aggregation (e.g., collection figures reference generation figures by district). A small number of aggregate cells in the compliance, informal-sector, environmental-monitoring, and awareness sheets were found to return zero pending formula recalculation in the source workbook; where this occurred, totals reported in this document were independently recomputed from the underlying row-level records to preserve accuracy.

III. Results

A. Generation & Inventory (Weight: 15; Score earned: 15.0)

Total e-waste generation across the twelve districts is estimated at 530,300 kg (530.3 t) for the reporting period. By category, IT & Telecom equipment contributes the largest share (235,600 kg, 44.4% of the total), followed by Consumer Electrical & Electronics (134,400 kg, 25.3%), Large & Small Equipment (86,100 kg, 16.2%), Batteries (36,700 kg, 6.9%), Lighting Equipment (22,200 kg, 4.2%), and Other categories (15,300 kg, 2.9%). Solan district reports the highest generation (103,200 kg, 19.5% of the state total), consistent with its industrial base, while Lahaul & Spiti reports the lowest (3,450 kg), reflecting its small population base. Table I summarizes district-wise figures.

TABLE I. DISTRICT-WISE E-WASTE GENERATION INVENTORY

| District | Population | IT & Telecom (kg) | Consumer Elec. (kg) | Large/Small Equip. (kg) | Lighting (kg) | Batteries (kg) | Other (kg) | Total (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bilaspur | 412,000 | 11,000 | 6,500 | 4,200 | 1,200 | 2,100 | 900 | 25,900 |
| Chamba | 525,000 | 12,000 | 7,200 | 4,600 | 1,300 | 2,200 | 1,000 | 28,300 |
| Hamirpur | 454,000 | 10,500 | 6,200 | 3,900 | 1,100 | 1,900 | 800 | 24,400 |
| Kangra | 1,565,000 | 36,000 | 20,000 | 13,000 | 3,200 | 5,200 | 2,100 | 79,500 |
| Kinnaur | 85,000 | 2,600 | 1,500 | 900 | 250 | 500 | 200 | 5,950 |
| Kullu | 465,000 | 14,500 | 8,600 | 5,200 | 1,400 | 2,500 | 1,000 | 33,200 |
| Lahaul & Spiti | 32,000 | 1,500 | 900 | 500 | 150 | 300 | 100 | 3,450 |
| Mandi | 1,020,000 | 22,500 | 13,000 | 8,500 | 2,100 | 3,400 | 1,500 | 51,000 |
| Shimla | 850,000 | 39,000 | 21,500 | 13,500 | 3,500 | 5,800 | 2,300 | 85,600 |
| Sirmaur | 580,000 | 19,000 | 11,000 | 6,800 | 1,800 | 2,900 | 1,300 | 42,800 |
| Solan | 580,000 | 46,000 | 26,000 | 17,500 | 4,200 | 6,800 | 2,700 | 103,200 |
| Una | 521,000 | 21,000 | 12,000 | 7,500 | 2,000 | 3,100 | 1,400 | 47,000 |
| State Total | 7,089,000 | 235,600 | 134,400 | 86,100 | 22,200 | 36,700 | 15,300 | 530,300 |

B. Collection Performance (Weight: 20; Score earned: 7.1)

Against total generation of 530,300 kg, the state collected 193,050 kg, a collection rate of 36.4%. Registered collection centers number 70 statewide across 12 districts. EPR collection targets aggregate to 188,400 kg, against which 177,120 kg was achieved (94.0% target achievement), indicating that producer-side EPR obligations are being substantially met even though overall collection relative to total generation remains comparatively low. The Baddi-Barotiwala-Nalagarh (BBN) industrial corridor in Solan district drives the state's highest collection volume (48,000 kg via 14 centers), while Lahaul & Spiti and Kinnaur, constrained by terrain, seasonal road closures, and low population density, report the lowest absolute collection volumes. Table II presents the full district-wise breakdown.

TABLE II. DISTRICT-WISE COLLECTION PERFORMANCE

| District | Generated (kg) | Collected (kg) | Centers | EPR Target (kg) | EPR Achieved (kg) | Collection Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Bilaspur | 25,900 | 8,500 | 4 | 8,000 | 7,400 | 32.8 |
| Chamba | 28,300 | 7,800 | 3 | 8,500 | 7,100 | 27.6 |
| Hamirpur | 24,400 | 8,200 | 4 | 7,500 | 7,300 | 33.6 |
| Kangra | 79,500 | 28,000 | 9 | 27,000 | 25,500 | 35.2 |
| Kinnaur | 5,950 | 700 | 1 | 1,200 | 600 | 11.8 |
| Kullu | 33,200 | 11,000 | 5 | 11,500 | 10,200 | 33.1 |
| Lahaul & Spiti | 3,450 | 350 | 1 | 700 | 320 | 10.1 |
| Mandi | 51,000 | 17,500 | 7 | 17,000 | 16,200 | 34.3 |
| Shimla | 85,600 | 32,000 | 10 | 31,000 | 29,500 | 37.4 |
| Sirmaur | 42,800 | 15,000 | 6 | 14,500 | 13,800 | 35.0 |
| Solan | 103,200 | 48,000 | 14 | 46,000 | 44,500 | 46.5 |
| Una | 47,000 | 16,000 | 6 | 15,500 | 14,700 | 34.0 |
| State Total | 530,300 | 193,050 | 70 | 188,400 | 177,120 | 36.4 |

C. Formal Recycling & Treatment (Weight: 20; Score earned: 20.0)

Four CPCB/SPCB-authorized recyclers/dismantlers currently operate in Himachal Pradesh, all located in Solan district within the BBN industrial corridor: M/s Shivalik Solid Waste Management (Unit-II), M/s Ortech India Corporations, M/s Eco Works Management Pvt. Ltd., and M/s Sahni Enterprise. Combined installed capacity is 6,360 tonnes/year, against which 1,182 tonnes were processed in the reporting year, with material recovery rates ranging from 75% to 82%. While this category scores fully against its benchmark, the complete geographic concentration of authorized recycling capacity in a single district means the remaining eleven districts depend entirely on inter-district transport for formal treatment.

TABLE III. AUTHORIZED RECYCLING INFRASTRUCTURE

| Recycler | District | Capacity (t/yr) | Processed (t) | Recovery (%) |
| --- | --- | --- | --- | --- |
| Shivalik SWM (Unit-II) | Solan | 5,200 | 3,900 | 82 |
| Ortech India Corp. | Solan | 500 | 340 | 78 |
| Eco Works Mgmt. | Solan | 540 | 410 | 80 |
| Sahni Enterprise | Solan | 120 | 78 | 75 |
| State Total | - | 6,360 | 1,182 | - |

D. Regulatory Compliance (Weight: 15; Score earned: 9.0)

Of five registered producer/EPR entities on file, three are fully compliant (60%), one is partially compliant (TransIndia Battery Producers Association, cited for a battery take-back shortfall), and one is non-compliant (SolanTech Lighting Solutions Pvt. Ltd., pending inspection with a penalty notice issued). Inspections were conducted for four of five planned cases (80%). Two penalty/show-cause notices have been issued to date, and none had been resolved as of the reporting date, indicating a gap between enforcement initiation and case closure.

TABLE IV. EPR COMPLIANCE SUMMARY

| Entity | District Basis | Status | Inspect. (P/C) | Penalty (Iss./Res.) |
| --- | --- | --- | --- | --- |
| Konnect Electronics Distrib. | Baddi | Compliant | Y/Y | N/- |
| Himgiri Appliances & Elec. | - | Compliant | Y/Y | N/- |
| TransIndia Battery Producers | - | Partial | Y/Y | Y/N |
| Nalagarh Industrial Electronics | Baddi-Nalagarh | Compliant | Y/Y | N/- |
| SolanTech Lighting Solutions | Solan | Non-Compliant | Y/N | Y/N |

E. Informal Sector Integration (Weight: 10; Score earned: 0.0)

Four informal e-waste clusters are recorded, of which three are in Solan district (Baddi Industrial Area Scrap Market, Nalagarh Informal Dismantling Cluster, Parwanoo Scrap Yard Cluster) and one in Shimla (Sabzi Mandi Scrap Dealers Cluster). Collectively, these hotspots involve an estimated 340 informal workers, of whom 127 (37.4%) have been registered or trained to date. No hotspot has reached full formalization status; classifications range from “Partially Formalized” and “Under Formalization” to “Identified” (baseline survey only, for the Shimla cluster). This category records the lowest achievement in the scorecard.

TABLE V. INFORMAL SECTOR HOTSPOTS

| Cluster | District | Status | Est. Workers | Registered |
| --- | --- | --- | --- | --- |
| Baddi Industrial Area Scrap Mkt. | Solan | Partially Formalized | 140 | 55 |
| Nalagarh Informal Dismantling | Solan | Under Formalization | 95 | 40 |
| Parwanoo Scrap Yard Cluster | Solan | Partially Formalized | 60 | 22 |
| Shimla Sabzi Mandi Cluster | Shimla | Identified | 45 | 10 |
| Total | - | - | 340 | 127 |

F. Environmental & Health Safeguards (Weight: 10; Score earned: 10.0)

Environmental monitoring was conducted at all four identified hotspots. Nalagarh Industrial Area drains and the Parwanoo Scrap Yard Cluster returned lead (Pb) and, in the former case, cadmium (Cd) levels marginally to clearly above permissible limits, while the Baddi Industrial Estate effluent channel and the Shimla Sabzi Mandi cluster returned results within permissible limits. Health screening camps were conducted at three of the four sites, collectively screening 335 beneficiaries (180 at Nalagarh, 95 at Parwanoo, 60 at Shimla); no camp was required at Baddi Industrial Estate given the routine, within-limits result.

TABLE VI. ENVIRONMENTAL AND HEALTH MONITORING

| Site | District | Result | Camp (Y/N) | Screened |
| --- | --- | --- | --- | --- |
| Nalagarh Industrial Drains | Solan | Pb, Cd above limit | Y | 180 |
| Baddi Estate Effluent Channel | Solan | Within limits | N | 0 |
| Parwanoo Scrap Yard Cluster | Solan | Pb above limit | Y | 95 |
| Shimla Sabzi Mandi Cluster | Shimla | Within limits | Y | 60 |
| Total | - | - | 3 camps | 335 |

G. Awareness & Outreach (Weight: 10; Score earned: 2.5)

Five awareness/outreach programs were conducted in the reporting period, collectively reaching approximately 3,475 participants: a school collection drive in Shimla (2,200 participants), an industrial EPR sensitization workshop in Solan (140), a scrap-collector integration drive in Baddi, Solan (85), a residents'-welfare-association collection camp in Shimla (650), and a tourist-belt clean-up and awareness drive in Kullu (400). Programs to date are concentrated in only three of twelve districts (Shimla, Solan, Kullu); the remaining nine districts have no recorded awareness activity, which is reflected in this category's low achievement against its benchmark.

TABLE VII. AWARENESS AND OUTREACH PROGRAMS

| Program | District | Type | Participants |
| --- | --- | --- | --- |
| Shimla Schools Collection Drive | Shimla | School Drive | 2,200 |
| BBN EPR Sensitization Workshop | Solan | Industry Workshop | 140 |
| Baddi Scrap Collector Integration | Solan | Informal Integration | 85 |
| RWA Community Collection Camp | Shimla | RWA/Community | 650 |
| Kullu-Manali Awareness Drive | Kullu | Public Campaign | 400 |
| Total | - | - | 3,475 |

IV. Discussion

The composite state score of 63.6/100 places Himachal Pradesh in the “Needs Improvement” band, one tier below “Good” (65-84). Two categories - Generation & Inventory and Formal Recycling & Treatment - achieve full marks, reflecting complete district reporting and recycler-side capacity utilization relative to their set benchmarks. However, Collection Performance (7.1/20), Informal Sector Integration (0/10), and Awareness & Outreach (2.5/10) are the weakest categories and, being collectively worth 40 of 100 points, are the principal drag on the overall score.

A recurring structural pattern is geographic concentration: all four authorized recyclers and three of four informal hotspots sit within Solan district's BBN corridor, while awareness programs cluster in Shimla, Solan, and Kullu. Hill and border districts - Kinnaur, Lahaul & Spiti, Chamba - consistently report the lowest collection volumes and center density, and have no recorded awareness activity, suggesting that infrastructure and outreach gaps are correlated with terrain and remoteness rather than generation volume alone. A secondary finding is an enforcement-to-resolution gap in regulatory compliance: while inspection coverage is reasonably high (80%), zero of two issued penalties have been resolved, which may weaken the deterrent effect of the compliance regime if left unaddressed.

V. Recommendations

Based on the findings above, five priority actions are proposed:

1) Expand registered collection-center density in hill and border districts (Kinnaur, Lahaul & Spiti, Chamba), including seasonal or mobile collection points to offset road-closure constraints.

2) Encourage establishment of authorized recycling/dismantling capacity outside the Solan/BBN corridor, or subsidize inter-district logistics, to reduce single-district dependency.

3) Institute time-bound penalty-resolution tracking for EPR non-compliance cases to close the gap between inspection and enforcement outcomes.

4) Prioritize full formalization (not partial integration) of informal hotspots through expanded worker registration, ID issuance, and skill-training targets, particularly at the Nalagarh and Shimla clusters.

5) Extend awareness and collection-drive coverage to the nine districts with no recorded programs, using the existing school- and RWA-drive formats as templates.

VI. Conclusion

The Himachal Pradesh E-Waste Management Tracker demonstrates a functioning district-level monitoring framework consistent with the E-Waste (Management) Rules, 2022. At a composite score of 63.6/100, the state performs strongly on generation reporting and formal recycling capacity utilization, but collection reach, informal-sector formalization, and awareness outreach lag behind and are concentrated in a small subset of districts. Targeted expansion of collection and outreach infrastructure in hill districts, diversification of recycling capacity beyond Solan, and closure of the compliance-enforcement loop represent the most direct paths toward moving the state from “Needs Improvement” into the “Good” band in subsequent reporting cycles.

References

[1] Ministry of Environment, Forest and Climate Change, “E-Waste (Management) Rules, 2022,” Government of India, Gazette Notification, 2022.

[2] Central Pollution Control Board (CPCB), “Guidelines for Environmentally Sound Management of E-Waste,” CPCB, New Delhi, India.

[3] Himachal Pradesh State E-Waste Management Tracker, internal district-level dataset, FY 2025-26 reporting cycle.